NaoTH - A framework by Berlin United used also by Dutch Nao Team.

A paper by Dutch Nao Team on how to improve NaoTH walking: https://staff.fnwi.uva.nl/a.visser/publications/WalkingOptimization.pdf

Advanced resources on other approaches to walking:

https://www.researchgate.net/publication/4041375_Biped_walking_pattern_generation_by_using_preview_control_of_zero-moment_point

https://www.researchgate.net/publication/251983299_Omnidirectional_closed-loop_walk_for_NAO

Walking script Berlin United

- LHipRoll and RHipRoll move in synchrony (y)

- LAnkleRoll and RAnkleRoll move at the same rate as HipRoll but inversed (-y)

- y is defined by a sin function multiplied by a shift value. If shift is above 0.1 (slow run #1) the robot falls after a couple steps

- KneePitch HipPitch and AnklePitch are all related. KneePitch is defined by z while HipPitch is (-z/2 + x) and AnklePitch is (-z/2 - x)

- zLeft is defined bya sin function plus 1, divided by 2 multiplied by step height plus robot height (sin(t) + 1/ (2 * step_height * robot -height)). zRight is defined the same way but the sin function has pi added to it sin(t + pi)

- x is defined by a cos function multiplied by step_length

Files Explanation:

- Walking.py works in WeBots, but not IRL

- Walking2.py tries to merge the functionality from Walking.py with the syntax from move_head.py, which is a file that works IRL for moving the robot’s head.

- Walking3.py uses a syntax that more strictly adheres to move_head, calling jointSensorData to read and jointMotorData to write, also self.update() instead of while True, and it works.

- Walking_official.py is a cleaner version we’re building, right off the Nao, without a Sprinter class