nano move_joints.py

LOLA = Low Level Abstractor, driver for RoboCup

They want to add a layer between LOLA and Python

RoboCup.conf

Temporary file that is empty, Lola creates a socket in it

Lola.py file can be found in the spl page, standard

Actuators and commands are structures

Commands are sent to the sockets

Read receives a message from a socket (896)

Main:

robot = Robot()

data = robot.read()

print(data)

Update function in NAO.py

Sends data to the robot, receives data and returns True

read_joints.py

Starts by setting joints stiffness to zero

If move joints by hand can print them in the while loop

Low stiffness (0.5) = can fuck with it a bit

Zero stiffness = can move it around, zero resistance

Webots platform to simulate

Same code, but different NAO class

pip3 install opencv-python-headless

Very important, because NAO has no monitor or display, so normal version will crash

opencv_naive.py

Opens a capture on the device zero (should be top camera)

Sleep for two seconds, as there's a couple of seconds of setup time

Reads one frame 480x640x3

Writes to file

scp very important tool (safe copy)

Opencv converts images to standardized format, but you can ask it for the raw

yuv422_test.py has cap.set to get the images faster from camera and in their format

I think resolution can be lowered by quite a bit

opencv_continuous.py reads videos

33ms between frames